DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CS T52 COMPUTER NETWORKS

V SEMESTER

SYLLABUS

CS T52 COMPUTER NETWORKS

UNIT – I

Introduction – Uses – Network hardware – software – reference models – example networks – Theoretical basis for communication – transmission media – wireless transmission – Communication  satellites

UNIT – II

Data link layer – design issues –  Services -  Framing - Error Control - Flow Control - Error detection and correction codes - data link layer protocols -Simplex Protocol – Sliding window Protocols - Medium Access control sublayer – Channel allocation problem – Multiple Access protocols – ALOHA – CSMA Protocols -  Collision-Free Protocols - Limited-Contention Protocols - Wireless LANs  -  802.11 Architecture -  802.16 Architecture – Data link layer Switching -  Uses of Bridges -  Learning Bridges -  Spanning Tree Bridges -  Repeaters, Hubs, Bridges, Switches, Routers, and Gateways -   Virtual LANs.

UNIT – III

Network layer – design issues – Routing algorithms -  The Optimality Principle - Shortest Path Algorithm – Flooding - Distance Vector Routing - Link State Routing - Hierarchical Routing - Broadcast Routing - Multicast Routing Congestion Control – Approaches - Traffic-Aware Routing - Admission Control - Traffic Throttling - Load Shedding – Internetworking -  Tunneling - Internetwork Routing -  Packet Fragmentation - IP v4 - IP Addresses – IPv6 -  Internet Control Protocols – OSPF - BGP

UNIT – IV

Transport layer -  Services - Berkeley Sockets -Example – Elements of Transport protocols – Addressing - Connection Establishment - Connection Release - Flow Control and Buffering – Multiplexing – Congestion Control - Bandwidth Allocation - Regulating the Sending Rate –UDP- RPC – TCP - TCP Segment Header - Connection Establishment - Connection Release - Transmission Policy - TCP Timer Management - TCP Congestion Control

UNIT – V

Application Layer – DNS – Name space – Resource records – name servers – e-mail - Architecture and Services - The User Agent - Message Formats - Message Transfer - Final Delivery – WWW – Architecture - Static Web Pages - Dynamic Web Pages and Web Applications -  HTTP – Network Security  - Introduction to Cryptography - Substitution Ciphers - Transposition Ciphers – Public key algorithms – RSA – Authentication Protocols - Authentication Using Kerberos.

Text Books:

Tanenbaum,A.S. and David J. Wetherall “Computer Networks”, 5th ed., Prentice Hall, 2011,

Reference Books:

- Larry L. Peterson and Bruce S. Davie, “Computer Networks- A system approach”, 5th edition, ELSEVIER, 2012

- Stallings, W., 'Data and Computer Communications', 10th Ed., Prentice Hall Int. Ed., 2013

- James F. Kurose and Keith W. Ross, “Computer Networking: A Top-Down Approach Featuring the Internet”, Pearson Education, Third edition, 2006.

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

Subject: COMPUTER NETWORKS                			Subject Code: CS T52

## UNIT I

Introduction – Uses – Network hardware – software – reference models – example networks – Theoretical basis for communication – transmission media – wireless transmission – Communication  satellites

- Define Computer Networks?

A computer network or data network is a telecommunications network which allows computers to exchange data. In computer networks, networked computing devices pass data to each other along network links (data connections). Data is transferred in the form of packets.

- What is Network? (November 2011)

A network is a group of two or more computer systems linked together. There are many types of computer networks, including the following:

- Local-area networks (LANs): The computers are geographically close together (that is, in the same building).

- Wide-area networks (WANs): The computers are farther apart and are connected by telephone lines or radio waves.

- Campus-area networks (CANs): The computers are within a limited geographic area, such as a campus or military base.

- Metropolitan-area networks MANs): A data network designed for a town or city.

- Home-area networks (HANs): A network contained within a user's home that connects a person's digital devices

- What is Middleware?

Software that acts as a bridge between an operating system or database and applications, especially on a network

- What is VPN?

VPN is a network that is constructed by using public wires (usually the Internet) to connect to a private network, such as a company's internal network. There are a number of systems that enable you to create networks using the Internet as the medium for transporting data.

- What is Server?

A computer or device on a network that manages network resources. There are many different types of servers. For example:

- File server: a computer and storage device dedicated to storing files. Any user on the network can store files on the server.

- Print server: a computer that manages one or more printers, and a network server is a computer that manages network traffic.

- Database server: a computer system that processes database queries.

- What is Client?

The client part of a client-server architecture. Typically, a client is an application that runs on a personal computer or workstation and relies on a server to perform some operations. For example, an e-mail client is an application that enables you to send and receive e-mail.

- What is Client Server Model?

The client–server model of computing is a distributed application structure that partitions tasks or workloads between the providers of a resource or service, called servers, and service requesters, called clients.

- What IP telephony?

## IP telephony (Internet Protocol telephony) is a general term for the technologies that use the Internet Protocol's packet-switched connections to exchange voice, fax, and other forms of information that have traditionally been carried over the dedicated circuit-switched connections of the public switched telephone network (PSTN).

- What are Cookies in Network?

A cookie is a mechanism that allows the server to store its own information about a user on the user's own computer. You can view the cookies that have been stored on your hard disk (although the content stored in each cookie may not make much sense to you). The location of the cookies depends on the browser.

- Define Broadcast, Multicast, Unicast?

- Unicast: A term used in communication to describe a piece of information to send from one point to another. There are only sender and receiver. All LANs support unicast transfer mode and most applications that employ TCP transport protocol uses unicast messaging.

- Broadcast: A term used for describing communication that is sent a piece of information from one point to all other points. There is one sender and multiple receivers. All LANs support broadcast transmission.

- Multicast: A term described in communicating a piece of information sent from one or more points to a set of other points. The senders and receivers are one or more

- What is resource sharing?

In computing, a shared resource, or network share, is a computer resource made available from one host to other hosts on a computer network.

- What is Phishing?

Phishing is an e-mail fraud method in which the perpetrator sends out legitimate-looking email in an attempt to gather personal and financial information from recipients.

- What is Bluetooth?

Bluetooth is defined as being a short-range radio technology (or wireless technology) aimed at simplifying communications among Internet devices and between devices and the Internet. It also aims to simplify data synchronization between Internet devices and other computers.

- Define LAN, WAN, PAN, and MAN.

LAN: A local area network (LAN) is a computer network that interconnects computers within a limited area such as a home, school, computer laboratory, or office building, using network media.

WAN: A wide area network (WAN) is a geographically dispersed telecommunications network. The term distinguishes a broader telecommunication structure from a local area network (LAN). A wide area network may be privately owned or rented, but the term usually connotes the inclusion of public (shared user) networks. An intermediate form of network in terms of geography is a metropolitan area network (MAN).

PAN: A personal area network (PAN) is a computer network used for data transmission among devices such as computers, telephones and personal digital assistants.

MAN: A metropolitan area network (MAN) is a network that interconnects users with computer resources in a geographic area or region larger than that covered by even a large local area network (LAN) but smaller than the area covered by a wide area network (WAN).

- What is enterprise Networks

An enterprise private network is a computer network built by a business to interconnect its various company sites (such as production sites, offices and shops) in order to share computer resources

- What is Access Point?

In computer networking, a wireless access point (AP) is a device that allows wireless devices to connect to a wired network using Wi-Fi, or related standards. The AP usually connects to a router (via a wired network) as a standalone device, but it can also be an integral component of the router itself.

- Define Ethernet?

a system for connecting a number of computer systems to form a local area network, with protocols to control the passing of information and to avoid simultaneous transmission by two or more systems.

- Define Switch, Router?

Switch: Switch. In networks, a device that filters and forwards packets between LAN segments. Switches operate at the data link layer (layer 2) and sometimes the network layer (layer 3) of the OSI Reference Model and therefore support any packet protocol.

Router: A router is a device that forwards data packets along networks. A router is connected to at least two networks, commonly two LANs or WANs or a LAN and its ISP's network. Routers are located at gateways, the places where two or more networks connect.

- Define Internet or internetwork?

Internetworking is the practice of connecting a computer network with other networks through the use of gateways that provide a common method of routing information packets between the networks. The resulting system of interconnected networks is called an internetwork, or simply an internet.

- Define Network Architecture?

Network architecture is the design of a communications network. It is a framework for the specification of a network's physical components and their functional organization and configuration, its operational principles and procedures, as well as data formats used in its operation.

- Define Protocol Stack?

The protocol stack is an implementation of a computer networking protocol suite. The terms are often used interchangeably. Strictly speaking, the suite is the definition of the protocols, and the stack is the software implementation of them.

- Define Error Detection and Error Correction

Error detection: Error detection is the detection of errors caused by noise or other impairments during transmission from the transmitter to the receiver. Error correction is the detection of errors and reconstruction of the original, error-free data.

Error correction: Error correction is the process of detecting errors in transmitted messages and reconstructing the original error-free data. Error correction ensures that corrected and error-free messages are obtained at the receiver side.

- Define Routing?

In internetworking, the process of moving a packet of data from source to destination. Routing is usually performed by a dedicated device called a router. Routing is a key feature of the Internet because it enables messages to pass from one computer to another and eventually reach the target machine. Each intermediary computer performs routing by passing along the message to the next computer. Part of this process involves analyzing a routing table to determine the best path.

- Define Addressing or Naming?

An IP address is a unique numerical value that is used to identify a computer on a network. There are two kinds of IP addresses, public (also called globally unique IP addresses) and private. Public IP addresses are assigned by the Internet Assigned Numbers Authority (IANA)

- What is statistical multiplexing?

Statistical multiplexing is a type of communication link sharing, very similar to dynamic bandwidth allocation (DBA). In statistical multiplexing, a communication channel is divided into an arbitrary number of variable bit-rate digital channels or data streams.

- What is Flow Control and Error Control?

- Flow control is the management of data flow between computers or devices or between nodes in a network so that the data can be handled at an efficient pace. Too much data arriving before a device can handle it causes data overflow, meaning the data is either lost or must be retransmitted.

- In information theory and coding theory with applications in computer science and telecommunication, error detection and correction or error control are techniques that enable reliable delivery of digital data over unreliable communication channels.

- Define Congestion

In data networking and queueing theory, network congestion occurs when a link or node is carrying so much data that its quality of service deteriorates. Typical effects include queueing delay, packet loss or the blocking of new connections.

- Define Packet

A piece of a message transmitted over a packet-switching network. See under packet switching. One of the key features of a packet is that it contains the destination address in addition to the data. In IP networks, packets are often called datagrams

- What is the difference between store-and-forward switching and cut-through switching?

- Store-and-Forward: Store-and-Forward switchingwill wait until the entire frame has arrived prior to forwardingit. This method stores the entire frame in memory. Once the frameis in memory, the switch checks the destination address, sourceaddress, and the CRC. If no errors are present, the frame isforwarded to the appropriate port. This process ensures that thedestination network is not affected by corrupted or truncatedframes.

- Cut-Through: Cut-Through switching will beginforwarding the frame as soon as the destination address isidentified. The difference between this and Store-and-Forward isthat Store-and-Forward receives the whole frame before forwarding.Since frame errors cannot be detected by reading only thedestination address, Cut-Through may impact network performance byforwarding corrupted or truncated frames. These bad frames cancreate broadcast storms wherein several devices on the networkrespond to the corrupted frames simultaneously.

- Define Datagram.

A datagram is a basic transfer unit associated with a packet-switched network. The delivery, arrival time, and order of arrival need not be guaranteed by the network.

- Define Internet Protocol (IP).

The Internet Protocol (IP) is the principal communications protocol in the Internet protocol suite for relaying datagrams across network boundaries. Its routing function enables internetworking, and essentially establishes the Internet.

- What is ICMP (Internet Control Message Protocol)?

The Internet Control Message Protocol (ICMP) is one of the main protocols of the Internet Protocol Suite. It is used by network devices, like routers, to send error messages indicating, for example, that a requested service is not available or that a host or router could not be reached.

- What is IMPs (Interface Message Processors?)

The Interface Message Processor (IMP) was the packet-switching node used to interconnect participant networks to the ARPANET from the late 1960s to 1989. It was the first generation of gateways, which are known today as routers.

- Define Network Socket.

A network socket is an endpoint of an inter-process communication across a computer network. Today, most communication between computers is based on the Internet Protocol; therefore most network sockets are Internet sockets.

- Define Sockets

A socket is one endpoint of a two-way communication link between two programs running on the network. A socket is bound to a port number so that the TCP layer can identify the application that data is destined to be sent to. An endpoint is a combination of an IP address and a port number.

- What is DSLAM (Digital Subscriber Line Access Multiplexer)?

A DSLAM (Digital Subscriber Line Access Multiplexer) is a network device, usually at a telephone company central office, that receives signals from multiple customer Digital Subscriber Line (DSL) connections and puts the signals on a high-speed backbone line using multiplexing techniques.

- What is CMTS (Cable Modem Termination System)?

A cable modem termination system or CMTS is a piece of equipment, typically located in a cable company's headend or hubsite, which is used to provide high speed data services, such as cable Internet or voice over Internet Protocol, to cable subscribers.

- Define Cable Modem.

A modem designed to operate over cable TV lines. Because the coaxial cable used by cable TV provides much greater bandwidth than telephone lines, a cable modem can be used to achieve extremely fast access to the World Wide Web. This, combined with the fact that millions of homes are already wired for cable TV, has made the cable modem something of a holy grail for Internet and cable TV companies.

- Define multipath Fading.

Multipath signals are received in a terrestrial environment, i.e., where different forms of propagation are present and the signals arrive at the receiver from transmitter via a variety of paths. Therefore there would be multipath interference, causing multi- path fading.

- What is RFCs (Request For Comments)

A Request for Comments (RFC) is a publication of the Internet Engineering Task Force (IETF) and the Internet Society, the principal technical development and standards-setting bodies for the Internet.

- Define Bandwidth.

In computing, bandwidth is the bit-rate of available or consumed information capacity expressed typically in metric multiples of bits per second. Variously, bandwidth may be characterized as network bandwidth, data bandwidth, or digital bandwidth.

- What is Full-duplex and Half-Duplex Links?

Full-duplex: Refers to the transmission of data in two directions simultaneously. For example, a telephone is a full-duplex device because both parties can talk at once. In contrast, a walkie-talkie is a half-duplex device because only one party can transmit at a time

Half-Duplex: Refers to the transmission of data in just one direction at a time. For example, a walkie-talkie is a half-duplex device because only one party can talk at a time. In contrast, a telephone is a full-duplex device because both parties can talk simultaneously. Duplex modes often are used in reference to network data transmissions.

- Define attenuation.

Attenuation is a general term that refers to any reduction in the strength of a signal. Attenuation occurs with any type of signal, whether digital or analog. Sometimes called loss, attenuation is a natural consequence of signal transmission over long distances.

- Define Multiplexing and De-Multiplexing.

Multiplexing: In telecommunications and computer networks, multiplexing (sometimes contracted to muxing) is a method by which multiple analog message signals or digital data streams are combined into one signal over a shared medium. The aim is to share an expensive resource.

De-Multiplexing: Demultiplex (DEMUX) is the reverse of the multiplex (MUX) process – combining multiple unrelated analog or digital signal streams into one signal over a single shared medium, such as a single conductor of copper wire or fiber optic cable. Thus, demultiplex is reconverting a signal containing multiple analog or digital signal streams back into the original separate and unrelated signals.

- Give any two design issues that occur in computer networking.(Apr 2011)

  - Connectivity

  - Cost-effective Resource Sharing

  - Support for common Services

  - Performance

- What is meant by Network?

A network is two or more nodes connected by a direct link, or two or more networks connected by one or more nodes”.

- Give any two advantages of packet switching? (April/May2012)

- They can provide variable data rates

- It is better for "bursty" traffic

- What are the two categories of physical media? Give an example for each category?(April/May 2012)

  - a. Guided Media

    - . Twisted –Pair cable

      - Shielded TP

      - Unshielded TP

    - Coaxial Cable

    - Fiber-optic cable

  - Unguided Media

    - Terrestrial microwave

    - Satellite Communication

- Write any four reasons for using layered protocols?

Design: In a layered model each layer is defined separately. Thus, the design problem is broken up into smaller and manageable pieces. Another advantage is it makes protocol designers to specialize in one area (or layer).

Change: When changes are made to one layer, it reduces the impact on the other layers. For example, protocol in one layer can be changed easily without if affecting higher or lower layers. If the models was not layered and consisted of a single layer then any change affects the entire model.

Learning: The layered approach divided a big more complex tack into several smaller tasks where each small task is performed by one layer. This makes it much easier to learn and understand the concept of each layer and the model.

Communication: The layered approach is useful for proper organizing and handling of communication. It also provides a standard programming interface between two layers.

Standards: It is the most important reason for using a layered model. A layered model provides a guideline and framework not a rigid standard to be used by the various vendors when creating their products. This is important for interoperability between the various vendors products hat perform different data communication tasks.

- What are the various types of network topology?

The most common nodes are computers and peripheral devices. Network topology is illustrated by showing these nodes and their connections using cables. There are a number of different types of network topologies, including point-to-point, bus, star, ring, mesh, tree and hybrid.

- Classify the network access?

Access networks can be loosely divided into three categories:

- Residential access networks, connecting a home end system into the network;

- Institutional access networks, connecting an end system in a business or educational institution into the network;

- Mobile access networks, connecting a mobile end system into the network

- What is transmission delay?    (November 2011)

Transmission Delay: Packet size / link bandwidth

Transmission delay is a function of the packet's length and has nothing to do with the distance between the two nodes. This delay is proportional to the packet's length in bits,

It is given by the following formula:

Seconds

Where

is the transmission delay in seconds

N is the number of bits, and

R is the rate of transmission (say in bits per second)

- What are the various types of services that the internet provides to its applications? (Apr 2013)

The two primary types of services which are made available by a particular network layer and which actually are also useful classifications for many non-technical types of service industries are known as connection-oriented and connectionless communication.

# What are the three categories of Access Networks?  	(November 2011)

Access networks can be loosely divided into three categories:

- residential access networks, connecting a home end system into the network;

- institutional access networks, connecting an end system in a business or educational institution into the network;

- mobile access networks, connecting a mobile end system into the network

- What is twisted pair? (April 2013)

It has two cables one to carry signal and the other for ground reference the difference of the two levels is taken by the receiving node. Cross talk and noise will be added to the two cables by the noise source. Noise is equal on both cables if they are twisted as the distance is same to both from the source of noise and unequal if parallel. So cables are twisted to avoid interference from outside.

- Marks

- What are the uses of computer networks? Explain with respective of real world applications.

- Business Applications

- Home Applications

- Mobile Users

- Social Issues

Business Applications

Most companies have a substantial number of computers. For example, a company may have a computer for each worker and use them to design products, write brochures, and do the payroll. Initially, some of these computers may have worked in isolation from the others, but at some point, management may have decided to connect them to be able to distribute information throughout the company. Put in slightly more general form, the issue here is resource sharing. The goal is to make all programs, equipment, and especially data available to anyone on the network without regard to the physical location of the resource or the user. An obvious and widespread example is having a group of office workers share a common printer. None of the individuals really needs a private printer, and a high-volume networked printer is often cheaper, faster, and easier to maintain than a large collection of individual printers. However, probably even more important than sharing physical resources such as printers, and tape backup systems, is sharing information. Companies small and large are vitally dependent on computerized information. Most companies have customer records, product information, inventories, financial statements, tax information, and much more online. If all of its computers suddenly went down, a bank could not last more than five minutes. A modern manufacturing plant, with a computer-controlled assembly line, would not last even 5 seconds. Even a small travel agency or three-person law firm is now highly dependent on computer networks for allowing employees to access relevant information and documents instantly.

- Networks called VPNs (Virtual Private Networks) may be used to join the individual networks at different sites into one extended network. In other words, the mere fact that a user happens to be 15,000 km away from his data should not prevent him from using the data as though they were local.

- The data are stored on powerful computers called servers. Often these are centrally housed and maintained by a system administrator. In contrast, the employees have simpler machines, called clients, on their desks, with which they access remote data, for example, to include in spreadsheets they are constructing.

A network with two clients and one server.

- This whole arrangement is called the client-server model. It is widely used and forms the basis of much network usage. The most popular realization is that of a Web application, in which the server generates Web pages based on its database in response to client requests that may update the database.

- The client-server model is applicable when the client and server are both in the same building (and belong to the same company), but also when they are far apart.

The client-server model involves requests and replies.

- Telephone calls between employees may be carried by the computer network instead of by the phone company. This technology is called IP telephony or Voice over IP (VoIP) when Internet technology is used.

- A third goal for many companies is doing business electronically, especially with customers and suppliers. This new model is called e-commerce (electronic commerce) and it has grown rapidly in recent years. Airlines, bookstores, and other retailers have discovered that many customers like the convenience of shop-ping from home. Consequently, many companies provide catalogs of their goods and services online and take orders online. Manufacturers of automobiles, aircraft, and computers, among others, buy subsystems from a variety of suppliers and then assemble the parts. Using computer networks, manufacturers can place orders electronically as needed. This reduces the need for large inventories and enhances efficiency.

Home Applications

- The biggest reason to buy a home computer was probably for Internet access. Now, many consumer electronic devices, such as set-top boxes, game consoles, and clock radios, come with embedded computers and computer networks, especially wireless networks, and home networks are broadly used for entertainment, including listening to, looking at, and creating music, photos, and videos.

- Internet access provides home users with connectivity to remote computers.  As with companies, home users can access information, communicate with other people, and buy products and services with e-commerce. The main benefit now comes from connecting outside of the home.

- Bob Metcalfe, the inventor of Ethernet, hypothesized that the value of a network is proportional to the square of the number of users because this is roughly the number of different connections that may be made (Gilder, 1993). This hypothesis is known as ‘‘Metcalfe’s law.’’

In a peer-to-peer system there are no fixed clients and servers

- Peer-to-peer communication is often used to share music and videos.

- The Internet can be used by applications to carry audio (e.g., Internet radio stations) and video (e.g., YouTube).

- Between person-to-person communications and accessing information are social network applications. Here, the flow of information is driven by the relationships that people declare between each other.

- One of the most popular social networking sites is Facebook. It lets people update their personal profiles and shares the updates with other people who they have declared to be their friends. Other social networking applications can make introductions via friends of friends, send news messages to friends such as Twitter above, and much more.

- Even more loosely, groups of people can work together to create content. A wiki, for example, is a collaborative Web site that the members of a community edit. The most famous wiki is the Wikipedia, an encyclopedia anyone can edit, but there are thousands of other wikis.

Some forms of e-commerce.

- Ubiquitous computing, in which computing is embedded into everyday life, as in the vision of Mark Weiser (1991). Many homes are already wired with security systems that include door and window sensors, and there are many more sensors that can be folded in to a smart home monitor, such as energy consumption.

- A technology called RFID (Radio Frequency IDentification) will push this idea even further in the future. RFID tags are passive (i.e., have no battery) chips the size of stamps and they can already be affixed to books, passports, pets, credit cards, and other items in the home and out. This lets RFID readers locate and Communicate with the items over a distance of up to several meters, depending on the kind of RFID. Originally, RFID was commercialized to replace barcodes. It has not succeeded yet because barcodes are free and RFID tags cost a few cents. Of course, RFID tags offer much more and their price is rapidly declining. They may turn the real world into the Internet of things (ITU, 2005).

Mobile Users

Mobile computers, such as laptop and handheld computers, are one of the fastest-growing segments of the computer industry. Their sales have already overtaken those of desktop computers. Why would anyone want one? People on the go often want to use their mobile devices to read and send email, tweet, watch movies, download music, play games, or simply to surf the Web for information. They want to do all of the things they do at home and in the office. Naturally, they want to do them from anywhere on land, sea or in the air.

Connectivity to the Internet enables many of these mobile uses. Since having a wired connection is impossible in cars, boats, and airplanes, there is a lot of interest in wireless networks. Cellular networks operated by the telephone companies are one familiar kind of wireless network that blankets us with coverage for mobile phones. Wireless hotspots based on the 802.11 standard are another kind of wireless network for mobile computers. They have sprung up everywhere that people go, resulting in a patchwork of coverage at cafes, hotels, airports, schools, trains and planes. Anyone with a laptop computer and a wireless modem can just turn on their computer on and be connected to the Internet through the hotspot, as though the computer were plugged into a wired network.

Combinations of wireless networks and mobile computing.

- Text messaging or texting is tremendously popular. It lets a mobile phone user type a short message that is then delivered by the cellular network to another mobile subscriber.

- Smart phones, such as the popular iPhone, combine aspects of mobile phones and mobile computers. The (3G and 4G) cellular networks to which they connect can provide fast data services for using the Internet as well as handling phone calls.

- Since mobile phones know their locations, often because they are equipped with GPS (Global Positioning System) receivers, some services are intentionally location dependent.

- When equipped with NFC (Near Field Communication) technology the mobile can act as an RFID smartcard and interact with a nearby reader for payment.

Social Issues

- Network neutrality

- DMCA takedown notices

- Gmail.

- Classify multiple processor systems by their rough physical size and Explain with neat Diagram. (Or) Explain the different types of Networks with neat diagram.

Classification of interconnected processors by scale

Personal Area Networks

- PANs (Personal Area Networks) let devices communicate over the range of a person. A common example is a wireless network that connects a computer with its peripherals.

- Almost every computer has an attached monitor, keyboard, mouse, and printer. Without using wireless, this connection must be done with cables.

- A short-range wireless network called Bluetooth to connect these components without wires.

Bluetooth PAN Configuration

Local Area Networks

- LAN (Local Area Network).

- A LAN is a privately owned network that operates within and nearby a single building like a home, office or factory.

- LANs are widely used to connect personal computers and consumer electronics to let them share resources (e.g., printers) and exchange information. When LANs are used by companies, they are called enterprise networks.

- In most cases, each computer talks to a device in the ceiling as shown in Fig. (a). This device, called an AP (Access Point), wireless router, or base station, relays packets between the wireless computers and also between them and the Internet.

- There is a standard for wireless LANs called IEEE 802.11, popularly known as WiFi, which has become very widespread. It runs at speeds anywhere from 11 to hundreds of Mbps.

Wireless and wired LANs (a) 802.11 (b) Switched Ethernet.

- The topology of many wired LANs is built from point-to-point links. IEEE 802.3, popularly called Ethernet, is, by far, the most common type of wired LAN. Fig. (b) shows a sample topology of switched Ethernet.

- Each computer speaks the Ethernet protocol and connects to a box called a switch with a point-to-point link. Hence the name. A switch has multiple ports, each of which can connect to one computer. The job of the switch is to relay packets between computers that are attached to it, using the address in each packet to determine which computer to send it to.

- The difficulty is how to carry both power and data signals at the same time. Part of the answer is that they use different frequency bands. In short, home LANs offer many opportunities and challenges. Most of the latter relate to the need for the networks to be easy to manage, dependable, and secure, especially in the hands of nontechnical users, as well as low cost.

Metropolitan Area Networks

- A MAN (Metropolitan Area Network) covers a city.

- The best-known examples of MANs are the cable television networks available in many cities.

- A MAN might look something like the system shown in Fig. In this figure we see both television signals and Internet being fed into the centralized cable headend for subsequent distribution to people’s homes.

Wide Area Networks

- A wide area network, or WAN, spans a large geographical area, often a country or continent. It contains of machines intended for running user (i.e., application) programs. We will follow traditional usage and call these machines hosts. The term end system is sometimes also used in the literature. The hosts are connected by a communication subnet, or just subnet for short. The job of the subnet is to carry messages from host to host, just as the telephone system carries words from speaker to listener. By separating the pure communication aspects of the network (the subnet) from the application aspects (the hosts), the complete network design is greatly simplified.

- In most wide area networks, the subnet consists of two distinct components: transmission lines & switching elements. Transmission lines (also called circuits, channels, or trunks) move bits between machines.

- The switching elements are specialized computers used to connect two or more transmission lines. When data arrive on an incoming line, the switching element must choose an outgoing line to forward them on.

- In most WANs, the network contains numerous cables or telephone lines, each one connecting a pair of routers. If two routers that do not share a cable nevertheless wish to communicate, they must do this indirectly, via other routers.  When a packet is send from one router to another via one or more intermediate routers, the packet is received at each intermediate router in its entirety, stored there until the required output line is free, and the forwarded.  A subnet using this principle is called a Point to Point, store and forward or packet switched subnet.  Nearly all wide area networks (except those using satellite) have store and forward subnet.  When the packet are small and all the same size, they often called cells.

A metropolitan area network based on cable TV

Internetworks

- Many networks exist in the world, often with different hardware and software.

- People connected to one network often want to communicate with people attached	to a different one.

- The fulfillment of this desire requires that different, and frequently incompatible, networks be connected.

- A collection of interconnected net works is called an internetwork or internet. These terms will be used in a generic sense, in contrast to the worldwide Internet (which is one specific internet), which we will always capitalize. The Internet uses ISP networks to connect enterprise networks, home networks, and many other networks. We will look at the Internet in great detail later in this book.

- Subnets, networks, and internetworks are often confused. The term ‘‘subnet’’ makes the most sense in the context of a wide area network, where it refers to the collection of routers and communication lines owned by the network operator.

- The general name for a machine that makes a connection between two or more networks and provides the necessary translation, both in terms of hardware and software, is a gateway.

- Gateways are distinguished by the layer at which they operate in the protocol hierarchy. We will have much more to say about layers and protocol hierarchies starting in the next section, but for now imagine that higher layers are more tied to applications, such as the Web, and lower layers are more tied to transmission links, such as Ethernet.

- Explain about Protocol Hierarchies with Necessary Diagram.

- To reduce their design complexity, most networks are organized as a stack of layers or levels, each one built upon the one below it. The number of layers, the name of each layer, the contents of each layer, and the function of each layer differ from network to network.

- The purpose of each layer is to offer certain services to the higher layers while shielding those layers from the details of how the Offered services are actually implemented. In a sense, each layer is a kind of virtual machine, offering certain services to the layer above it.

- This concept is actually a familiar one and is used throughout computer science, where it is variously known as information hiding, abstract data types, data encapsulation, and object-oriented programming. The fundamental idea is that a particular piece of software (or hardware) provides a service to its users but keeps the details of its internal state and algorithms hidden from them.

Figure. Layers, protocols, and interfaces

A five-layer network is illustrated in Fig. The entities comprising the corresponding layers on different machines are called peers. In reality, no data are directly transferred from layer n  on one machine to layer n  on another machine. Instead, each layer passes data and control information to the layer immediately below it, until the lowest layer is reached.

- Above layer 1 is the physical medium through which actual communication occurs. In Fig, virtual communication is shown by dotted lines and physical communication by solid lines. Between each pair of adjacent layers is an interface.

- The interface defines which primitive operations and services the lower layer makes available to the upper one. When network designers decide how many layers to include in a network and what each one should do, one of the most important considerations is defining clean interfaces between the layers. Doing so, in turn, requires that each layer perform a specific collection of well-understood functions. In addition to minimizing the amount of information that must be passed between layers, clear-cut interfaces also make it simpler to replace one layer with a completely different protocol or implementation (e.g., replacing all the telephone lines by satellite channels) because all that is required of the new protocol or implementation is that it offer exactly the same set of services to its upstairs neighbor as the old one did.

- It is common that different hosts use different implementations of the same protocol (often written by different companies). In fact, the protocol itself can change in some layer without the layers above and below it even noticing.

- A set of layers and protocols is called network architecture. The specification of architecture must contain enough information to allow an implementer to write the program or build the hardware for each layer so that it will correctly obey the appropriate protocol. Neither the details of the implementation nor the specification of the interfaces is part of the architecture because these are hidden away inside the machines and not visible from the outside. It is not even necessary that the interfaces on all machines in a network be the same, provided that each machine can correctly use all the protocols. A list of the protocols used by a certain system, one protocol per layer, is called a protocol stack.

Figure: The philosopher-translator-secretary architecture

Figure: Example information flow supporting virtual communication in layer 5

The peer process abstraction is crucial to all network design. Using it, the unmanageable task of designing the complete network can be broken into several smaller, manageable design problems, namely, the design of the individual layers.

- Discuss about what are the Design Issues for the Layers.

Some of the key design issues that occur in computer networks will come up in layer after layer. Below, we will briefly mention the more important ones.

- Reliability is the design issue of making a network that operates correctly even though it is made up of a collection of components that are themselves unreliable. Think about the bits of a packet traveling through the network.

- There is a chance that some of these bits will be received damaged (inverted) due to fluke electrical noise, random wireless signals, hardware flaws, software bugs and so on. How is it possible that we find and fix these errors?  One mechanism for finding errors in received information uses codes for error detection. Information that is incorrectly received can then be retransmitted until it is received correctly. More powerful codes allow for error correction, where the correct message is recovered from the possibly incorrect bits that were originally received. Both of these mechanisms work by adding redundant information. They are used at low layers, to protect packets sent over individual links, and high layers, to check that the right contents were received.

- Another reliability issue is finding a working path through a network. Often there are multiple paths between a source and destination, and in a large network, here may be some links or routers that are broken.

- A second design issue concerns the evolution of the network. Over time, networks grow larger and new designs emerge that need to be connected to the existing network. We have recently seen the key structuring mechanism used to support change by dividing the overall problem and hiding implementation details: protocol layering. There are many other strategies as well. Since there are many computers on the network, every layer needs a mechanism for identifying the senders and receivers that are involved in a particular message. This mechanism is called addressing or naming, in the low and high layers, respectively. An aspect of growth is that different network technologies often have different limitations.

- Designs that continue to work well when the network gets large are said to be scalable.

- A third design issue is resource allocation. Networks provide a service to hosts from their underlying resources, such as the capacity of transmission lines. To do this well, they need mechanisms that divide their resources so that one host does not interfere with another too much. Many designs share network bandwidth dynamically, according to the short-term needs of hosts, rather than by giving each host a fixed fraction of the bandwidth that it may or may not use. This design is called statistical multiplexing, meaning sharing based on the statistics of demand. It can be applied at low layers for a single link, or at high layers for a network or even applications that use the network.

- An allocation problem that occurs at every level is how to keep a fast sender from swamping a slow receiver with data. Feedback from the receiver to the sender is often used. This subject is called flow control. Sometimes the problem is that the network is oversubscribed because too many computers want to send too much traffic, and the network cannot deliver it all. This overloading of the network is called congestion. One strategy is for each computer to reduce its demand when it experiences congestion. It, too, can be used in all layers. It is interesting to observe that the network has more resources to offer than simply bandwidth. For uses such as carrying live video, the timeliness of delivery matters a great deal. Most networks must provide service to applications that want this real-time delivery at the same time that they provide service to applications that want high throughput. Quality of service is the name given to mechanisms that reconcile these competing demands. The last major design issue is to secure the network by defending it against different kinds of threats. One of the threats we have mentioned previously is that of eavesdropping on communications. Mechanisms that provide confidentiality defend against this threat, and they are used in multiple layers. Mechanisms for authentication prevent someone from impersonating someone else.

- Write short notes on Connection Oriented and Connection less Services. (or) Compare Connection Oriented versus Connection Less Services. (November 2011/12)

A Service Description

Connectionless and Connection-Oriented Services

The two primary types of services which are made available by a particular network layer and which actually are also useful classifications for many non-technical types of service industries are known as connection-oriented and connectionless communication.

Connection-oriented Services

One of the easiest ways to understand what a connection-oriented protocol is would be to think of a very familiar service upon which it's based: the telephone system. When I pick up the phone, I have an open circuit, and the dial tone carrier signal allows me to connect to a destination of my choosing.

Given valid input parameters, the service:

- Establishes the connection.

- Allows me to utilize the connection.

- Tears down the connection when I'm done using it

The primary difference between this method and that of a connectionless service is that in a connection-oriented system, all of my communications are taking place on the same transmission channel. On the other hand, with a connectionless service, all transmissions are independently routed, and perhaps re-assembled in some order at the other end -- the service in between has no inherent responsibility for ensuring ordinarily -- it need only assure that each transmission gets delivered from its source to its destination.

Connectionless Services

A good analogy for a connectionless service is the process of sending letters through the postal system. Each transmission (the "letter") contains the full destination address and is processed independent of related messages. As described above, the service has only to ensure that each reaches its host within certain time parameters. Unlike a connection-oriented service, the system has free reign on what happens enroute between the sender and receiver:

- A message can be delayed to ensure another arrives first.

- Widely different channels of communication can be used for transmitting messages.

- A message can be handed off to a trusted third party in the distribution network.

- A message can be intercepted by a third party, copied or logged, and passed on to the intended receiver.

These operations are basically impossible for a connection-oriented service.

Figure: Six different types of service

Types of Services Available in TCP/IP and OSI:

The OSI Reference Model provides for both connection-oriented and connectionless communication at the network level. However, it supports only connection-oriented communication at the Transport layer. It became obvious after the initial design of OSI that allowing both types of traffic at the transport layer was important even though it violated the idea of data abstraction which was central to the design of OSI.

TCP/IP, on the other hand, supports only connectionless traffic at the Network layer, but supports both modes in the Transport layer. This allows for simple request-response protocols to be easily implemented, though it complicates things somewhat for the user. At the Transport level, TCP, the Transmission Control Protocol (a connection-oriented service) as well as UDP, Datagram Protocol (a connectionless service) are provided.

Quality of Service

Reliability of connections achieved through connectionless and connection-oriented protocols is another major concern. All protocols are not created equal, and sacrifices in reliability can be made in exchange for greater speed, or vice versa. Often the trade-offs are worth it, assuming that we're attempting to fit our task intelligently into the capacities of the protocol.

Sometimes it's necessary for a "handshake" process to occur, especially if we need to authenticate each piece of traffic from a sender, but in many cases (such as streaming video), the performance hit involved is simply unacceptable. Not all applications require connections, and it's naive to think of either protocol as superior to the other. In certain cases, it's not even necessary to ensure that a message gets sent, so long as the chance that it was received is high enough (think of the high-volume email transmissions of spammers). Consider how difficult and time-consuming it would be if each spam message had to be acknowledged by the receiver and tracked by the spammer.

- Briefly explain circuit switching and packet switching? (November 2011) (April 2013)

After getting the media for transmission it is not possible to use a continuous media to the end point .So we connect different links by a switch to the required destination by going through more than one switch according to requirement. So here, we have switching technique implemented to divert (route) the line on to required direction and a media can be used by different people to communicate. So a switch does this function to connect multiple communication lines from different locations. There are different techniques to implement it they are as follows:

Circuit switching:

This switching technique is done by establishing dedicated lines and all information from a source to destination go on the same path that is the dedicated path and no sequencing is required.

- Circuit Switching is the simplest method of data communication. Landline telephone network uses this method to establish connection between two subscribers.

- Circuit switching establishes a physical connection between two devices and the physical path that is established remains open until the end of the session.

- At any point of time only two communication devices can transmit information to each other. These devices could be either telephones or computers.

- Once a circuit is established between two stations, it is exclusively used by the two parties and the dedicated link becomes unavailable to others till the call is terminated by either party.

- This will be the experience of every person using the telephone. We make a call and either we get our destination party or encounter a busy signal.

- A single circuit is used for the entire duration of call.

Packet switching: (N02 - 1M)

Here long messages are broken into smaller ones called packets and transmit on a link. The transmission on a link for a required route is done when it is available by storing in a switch. A link existing between two switches is shared to transmit packets by different pair of source and destination. Here packets from the same source to a destination go on different links and does not reach the destination in the same sequence as transmitted so sequencing of packets to reassemble at destination is required. Packet switching is implemented in two different ways they are as follows

- Datagram Packet switching.

- Virtual circuit switching.

Datagram Packet switching

Here the packets are transmitted independent of each other in different paths. The packet is called as datagram. The Fig shows how packets go from A to X.

Virtual circuit switching

A logical circuit is established between a sender and receiver on a handshake and all packets are transmitted in the same path as shown in Fig from C to B. The following fig shows the transmission in both cases

Message switching: (RTP)

- Message switching is a system which uses computer techniques to transmit, receive, store and retrieve textual information.

- A message is a logical unit of information and can be of any length. In this method, if a station wants to send a message to another station, the destination address is attached to the message and then transmitted to the destination through intermediary nodes.

- The intermediary node receives the message, stores it temporarily, inspects for errors and transmits the message to the next node on the basis of availability of channel. This process goes on till the message reaches its intended destination.

- Organizations with heavy volume of data transmission use this method.

- It causes long delay because the message is stored at the exchange until suitable circuits are available to transmit message.

- For message switching it is not necessary to establish a dedicated path between 2 stations.

- Examples are telegrams, electronic mail, computer files, transaction queries and responses.

- Explain OSI Reference Model in Detail with neat diagram.

ISO defines a common way to connect computer by the architecture called Open System Interconnection (OSI) architecture. Network functionality is divided into seven layers.

Organization of the layers: The 7 layers can be grouped into 3 subgroups

- Network Support Layers

Layers 1,2,3 - Physical, Data link and Network are the network support layers. They deal with the physical aspects of moving data from one device to another such as electrical specifications, physical addressing, transport timing and reliability.

- Transport Layer

Layer4, transport layer, ensures end-to-end reliable data transmission on a single link.

- User Support Layers

Layers 5,6,7 – Session, presentation and application are the user support layers. They allow interoperability among unrelated software systems

An Data exchange using the OSI model

Functions of the Layers

1. Physical Layer

The physical layer coordinates the functions required to transmit a bit stream over a physical medium.

The physical layer is concerned with the following:

- Physical characteristics of interfaces and media - The physical layer defines the characteristics of the interface between the devices and the transmission medium.

- Representation of bits - To transmit the stream of bits, it must be encoded to signals. The physical layer defines the type of encoding.

- Data Rate or Transmission rate - The number of bits sent each second – is also defined by the physical layer.

- Synchronization of bits - The sender and receiver must be synchronized at the bit level. Their clocks must be synchronized.

- Line Configuration - In a point-to-point configuration, two devices are connected together through a dedicated link. In a multipoint configuration, a link is shared between several devices.

- Physical Topology - The physical topology defines how devices are connected to make a network. Devices can be connected using a mesh, bus, star or ring topology.

- Transmission Mode - The physical layer also defines the direction of transmission between two devices: simplex, half-duplex or full-duplex.

2. Data Link Layer

It is responsible for transmitting frames from one node to next node.

The other responsibilities of this layer are

- Framing - Divides the stream of bits received into data units called frames.

- Physical addressing – If frames are to be distributed to different systems on the n/w , data link layer adds a header to the frame to define the sender and receiver.

- Flow control- If the rate at which the data are absorbed by the receiver is less than the rate produced in the sender ,the Data link layer imposes a flow ctrl mechanism.

- Error control- Used for detecting and retransmitting damaged or lost frames and to prevent duplication of frames. This is achieved through a trailer added at the end of the frame.

- Access control -Used to determine which device has control over the link at any given time.

3. NETWORK LAYER (Apr 2015)

This layer is responsible for the delivery of packets from source to destination.

It is mainly required, when it is necessary to send information from one network to another.

The other responsibilities of this layer are

- Logical addressing - If a packet passes the n/w boundary, we need another addressing system for source and destination called logical address.

- Routing – The devices which connects various networks called routers are responsible for delivering packets to final destination.

4. TRANSPORT LAYER

- It is responsible for Process to Process delivery.

- It also ensures whether the message arrives in order or not.

The other responsibilities of this layer are

- Port addressing - The header in this must therefore include a address called port address. This layer gets the entire message to the correct process on that computer.

- Segmentation and reassembly - The message is divided into segments and each segment is assigned a sequence number. These numbers are arranged correctly on the arrival side by this layer.

- Connection control - This can either be connectionless or connection oriented. The connectionless treats each segment as a individual packet and delivers to the destination. The connection-oriented makes connection on the destination side before the delivery. After the delivery the termination will be terminated.

- Flow and error control - Similar to data link layer, but process to process take place.

5. SESSION LAYER (Apr 2015)

This layer establishes, manages and terminates connections between applications.

The other responsibilities of this layer are

- Dialog control - This session allows two systems to enter into a dialog either in half duplex or full duplex.

- Synchronization-This allows to add checkpoints into a stream of data.

6. PRESENTATION LAYER

It is concerned with the syntax and semantics of information exchanged between two systems.

The other responsibilities of this layer are

- Translation – Different computers use different encoding system, this layer is responsible for interoperability between these different encoding methods. It will change the message into some common format.

- Encryption and decryption-It means that sender transforms the original information to another form and sends the resulting message over the n/w. and vice versa.

- Compression and expansion-Compression reduces the number of bits contained in the information particularly in text, audio and video.

7. APPLICATION LAYER

This layer enables the user to access the n/w. This allows the user to log on to remote user.

The other responsibilities of this layer are

- FTAM (file transfer, access, mgmt) - Allows user to access files in a remote host.

- Mail services - Provides email forwarding and storage.

- Directory services - Provides database sources to access information about various sources and objects.

- Discuss about TCP/IP Reference Model with neat diagram. (Or) Draw the ISO-OSI reference model and explain the functionalities of each layer in Detail. (April/May 2012)

Let us now turn from the OSI reference model to the reference model used in the grandparent of all wide area computer networks, the ARPANET, and its successor, the worldwide Internet. Although we will give a brief history of the ARPANET later, it is useful to mention a few key aspects of it now.

The ARPANET was a research network sponsored by the DoD (U.S. Department of Defense). It eventually connected hundreds of universities and government installations, using leased telephone lines. When satellite and radio networks were added later, the existing protocols had trouble interworking with them, so a new reference architecture was needed. Thus, from nearly the beginning, the ability to connect multiple networks in a seamless way was one of the major design goals.

This architecture later became known as the TCP/IP Reference Model, after its two primary protocols. It was first described by Cerf and Kahn (1974), and later refined and defined as a standard in the Internet community (Braden, 1989). The design philosophy behind the model is discussed by Clark (1988). Given the DoD’s worry that some of its precious hosts, routers, and internetwork gateways might get blown to pieces at a moment’s notice by an attack from the Soviet Union, another major goal was that the network be able to survive loss of subnet hardware, without existing conversations being broken off. In other words, the DoD wanted connections to remain intact as long as the source and destination machines were functioning, even if some of the machines or transmission lines in between were suddenly put out of operation. Furthermore, since applications with divergent requirements were envisioned, ranging from transferring files to real-time speech transmission, a flexible architecture was needed.

The Link Layer

All these requirements led to the choice of a packet-switching network based on a connectionless layer that runs across different networks. The lowest layer in the model, the link layer describes what links such as serial lines and classic Ethernet must do to meet the needs of this connectionless internet layer. It is not really a layer at all, in the normal sense of the term, but rather an interface between hosts and transmission links. Early material on the TCP/IP model has little to say about it.

The Internet Layer

The internet layer is the linchpin that holds the whole architecture together. It is shown in Fig. 1-21 as corresponding roughly to the OSI network layer. Its job is to permit hosts to inject packets into any network and have them travel independently to the destination (potentially on a different network). They may even arrive in a completely different order than they were sent, in which case it is the job of higher layers to rearrange them, if in-order delivery is desired. Note that ‘‘internet’’ is used here in a generic sense, even though this layer is present in the Internet

Figure. The TCP/IP reference model

The analogy here is with the (snail) mail system. A person can drop a sequence of international letters into a mailbox in one country, and with a little luck, most of them will be delivered to the correct address in the destination country. The letters will probably travel through one or more international mail gateways along the way, but this is transparent to the users. Furthermore, that each country (i.e., each network) has its own stamps, preferred envelope sizes, and delivery rules is hidden from the users. The internet layer defines an official packet format and protocol called IP (Internet Protocol), plus a companion protocol called ICMP (Internet Control Message Protocol) that helps it function. The job of the internet layer is to deliver IP packets where they are supposed to go. Packet routing is clearly a major issue here, as is congestion (though IP has not proven effective at avoiding congestion).

The Transport Layer

The layer above the internet layer in the TCP/IP model is now usually called the transport layer. It is designed to allow peer entities on the source and destination hosts to carry on a conversation, just as in the OSI transport layer. Two end-to-end transport protocols have been defined here.

- The first one, TCP (Transmission Control Protocol), is a reliable connection-oriented protocol that allows a byte stream originating on one machine to be delivered without error on any other machine in the internet. It segments the incoming byte stream into discrete messages and passes each one on to the internet layer. At the destination, the receiving TCP process reassembles the received messages into the output stream. TCP also handles flow control to make sure a fast sender cannot swamp a slow receiver with more messages than it can handle.

- The second protocol in this layer, UDP (User Datagram Protocol), is unreliable, connectionless protocols for applications that do not want TCP’s sequencing or flow control and wish to provide their own. It is also widely used for one-shot, client-server-type request-reply queries and applications in which prompt delivery is more important than accurate delivery, such as transmitting speech or video. The relation of IP, TCP, and UDP is shown in Fig. Since the model was developed, IP has been implemented on many other networks.

The Application Layer

The TCP/IP model does not have session or presentation layers. No need for them was perceived. Instead, applications simply include any session and presentation functions that they require. Experience with the OSI model has proven this view correct: these layers are of little use to most applications.  On top of the transport layer is the application layer. It contains all the higher-level protocols. The early ones included virtual terminal (TELNET), file transfer (FTP), and electronic mail (SMTP).

- Compare the OSI Reference Model and TCP/IP Reference Model.

The OSI and TCP/IP models are having many similarities in the functionalities provided by the layers. The layers of TCP model behave similar to the layers of OSI model. But these two models do have differences.

Three concepts are central to the OSI model:

- Services.

- Interfaces.

- Protocols.

- Probably the biggest contribution of the OSI model is that it makes the distinction between these three concepts explicit. Each layer performs some services for the layer above it. The service definition tells what the layer does, not how entities above it access it or how the layer works. It defines the layer’s semantics.

- A layer’s interface tells the processes above it how to access it. It specifies what the parameters are and what results to expect. It, too, says nothing about how the layer works inside.

- The peer protocols used in a layer are the layer’s own business. It can use any protocols it wants to, as long as it gets the job done (i.e., provides the offered services). It can also change them at will without affecting software in higher layers.

Similarities:

The main similarities between the two models include the following:

- They share similar architecture. -    Both of the models share a similar architecture.  This can be illustrated by the fact that both of them are constructed with layers.

- They share a common application layer.- Both of the models share a common "application layer".  However in practice this layer includes different services depending upon each model.

- Both models have comparable transport and network layers- This can be illustrated by the fact that whatever functions are performed between the presentation and network layer of the OSI model similar functions are performed at the Transport layer of the TCP/IP model.

- Both models assume that packets are switched- Basically this means that individual packets may take differing paths in order to reach the same destination.

Differences:

- TCP/IP Protocols are considered to be standards around which the internet has developed.  The OSI model however is a "generic, protocol- independent standard.“

- TCP/IP combines the presentation and session layer issues into its application layer.

- TCP/IP combines the OSI data link and physical layers into the network access layer.

- TCP/IP appears to be a more simpler model and this is mainly due to the fact that it has fewer layers.

- TCP/IP is considered to be a more credible model- This is mainly due to the fact because TCP/IP protocols are the standards around which the internet was developed therefore it mainly gains creditability due to this reason.  Where as in contrast networks are not usually built around the OSI model as it is merely used as a guidance tool.

- The OSI model consists of 7 architectural layers whereas the TCP/IP only has 4 layers.

Comparison:

| OSI Model | TCP/IP Model |
| --- | --- |
| OSI stands for Open System Interconnection because it allows any two different systems to communicate regardless of their architecture. | TP/IP stands for Transmission Control Protocol/Internet Protocol. It is named after these protocols, being part of this model. |
| OSI model has seven layers. | TCP/IP has four layers. |
| This model provides clear distinction between services, interfaces and protocols | It does not clearly distinguish between services, interfaces & protocols. |
| In this model, Protocols do not fit well into the model. | TCP and IP protocols fit well in the model. |
| Session & Presentation layers are present in this layer. | There is no session & presentation layer in this model |
| OSI model supports both connection oriented & connectionless in network layer but connection oriented comm. In transport layer. | TCP/IP supports only connectionless comm. In network layer but supports both in transport layer. |

- List and briefly explain about the Critique of OSI Reference Model and TCP/IP Reference Model.

Critique of OSI Reference Model

Neither the OSI model and its protocols nor the TCP/IP model and its protocols are perfect. Quite a bit of criticism can be, and has been, directed at both of them. In this section and the next one, we will look at some of these criticisms. We will begin with OSI and examine TCP/IP afterward. At the time the second edition of this book was published (1989), it appeared to many experts in the field that the OSI model and its protocols were going to take over the world and push everything else out of their way. This did not happen. Why? A look back at some of the reasons may be useful. They can be summarized as:

- Bad timing.

- Bad technology.

- Bad implementations.

- Bad politics.

Critique of TCP/IP Reference Model

- The TCP/IP model and protocols have their problems too. First, the model does not clearly distinguish the concepts of services, interfaces, and protocols. Good software engineering practice requires differentiating between the specification and the implementation, something that OSI does very carefully, but TCP/IP does not. Consequently, the TCP/IP model is not much of a guide for designing new networks using new technologies.

- Second, the TCP/IP model is not at all general and is poorly suited to describing any protocol stack other than TCP/IP. Trying to use the TCP/IP model to describe Bluetooth, for example, is completely impossible.

- Third, the link layer is not really a layer at all in the normal sense of the term as used in the context of layered protocols. It is an interface (between the network and data link layers). The distinction between an interface and a layer is crucial, and one should not be sloppy about it. Fourth, the TCP/IP model does not distinguish between the physical and data link layers.

These are completely different. The physical layer has to do with the transmission characteristics of copper wire, fiber optics, and wireless communication. The data link layer’s job is to delimit the start and end of frames and get them from one side to the other with the desired degree of reliability. A proper model should include both as separate layers. The TCP/IP model does not do this. Finally, although the IP and TCP protocols were carefully thought out and well implemented, many of the other protocols were ad hoc, generally produced by a couple of graduate students hacking away until they got tired. The protocol implementations were then distributed free, which resulted in their becoming widely used, deeply entrenched, and thus hard to replace. Some of them are a bit of an embarrassment now. The virtual terminal protocol, TELNET, for example, was designed for a ten-character-per-second mechanical Teletype terminal.

- Draw the Architecture of internet mention the components incorporated in the Architecture.

The architecture of the Internet has also changed a great deal as it has grown explosively. The picture is complicated by continuous upheavals in the businesses of telephone companies (telcos), cable companies and ISPs that often make it hard to tell who is doing what. One driver of these upheavals is telecommunications convergence, in which one network is used for previously different uses. For example, in a ‘‘triple play’’ one company sells you telephony, TV, and Internet service over the same network connection on the assumption that this will save you money. Consequently, the description given here will be of necessity somewhat simpler than reality. And what is true today may not be true tomorrow.

The big picture is shown in Fig. Let us examine this figure piece by piece, starting with a computer at home (at the edges of the figure). To join the Internet, the computer is connected to an Internet Service Provider, or simply ISP, from who the user purchases Internet access or connectivity. This lets the computer exchange packets with all of the other accessible hosts on the Internet. The user might send packets to surf the Web or for any of a thousand other uses, it does not matter. There are many kinds of Internet access, and they are usually distinguished by how much bandwidth they provide and how much they cost, but the most important attribute is connectivity.

Figure. Overview of the Internet architecture

- A common way to connect to an ISP is to use the phone line to your house, in which case your phone company is your ISP.

- DSL, short for Digital Subscriber Line, reuses the telephone line that connects to your house for digital data transmission. The computer is connected to a device called a DSL modem that converts between digital packets and analog signals that can pass unhindered over the telephone line. At the other end, a device called a DSLAM (Digital Subscriber Line Access Multiplexer) converts between signals and packets.

- Several other popular ways to connect to an ISP are shown in Fig. DSL is a higher-bandwidth way to use the local telephone line than to send bits over a traditional telephone call instead of a voice conversation. That is called dial-up and done with a different kind of modem at both ends. The word modem is short for ‘‘ mo dulator dem odulator’’ and refers to any device that converts between digital bits and analog signals. Another method is to send signals over the cable TV system. Like DSL, this is a way to reuse existing infrastructure, in this case otherwise unused cable TV channels.

- The device at the home end is called a cable modem and the device at the cable headend is called the CMTS (Cable Modem Termination System). DSL and cable provide Internet access at rates from a small fraction of a megabit/sec to multiple megabit/sec, depending on the system. These rates are much greater than dial-up rates, which are limited to 56 kbps because of the narrow bandwidth used for voice calls. Internet access at much greater than dial-up speeds is called broadband.

- The name refers to the broader bandwidth that is used for faster networks, rather than any particular speed. The access methods mentioned so far are limited by the bandwidth of the ‘‘last mile’’ or last leg of transmission. By running optical fiber to residences, faster Internet access can be provided at rates on the order of 10 to 100 Mbps. This design is called FTTH (Fiber to the Home).

- For businesses in commercial areas, it may make sense to lease a high-speed transmission line from the offices to the nearest ISP. For example, in North America, a T3 line runs at roughly 45 Mbps. Wireless is used for Internet access too. An example we will explore shortly is that of 3G mobile phone networks. They can provide data delivery at rates of 1

- Mbps or higher to mobile phones and fixed subscribers in the coverage area. We can now move packets between the home and the ISP. We call the location at which customer packets enter the ISP network for service the ISP’s POP (Point of Presence). We will next explain how packets are moved between the POPs of different ISPs. From this point on, the system is fully digital and packet switched.

- ISP networks may be regional, national, or international in scope. We have already seen that their architecture is made up of long-distance transmission lines that interconnect routers at POPs in the different cities that the ISPs serve. This equipment is called the backbone of the ISP. If a packet is destined for a host served directly by the ISP, that packet is routed over the backbone and delivered to the host. Otherwise, it must be handed over to another ISP. ISPs connect their networks to exchange traffic at IXPs (Internet eXchange Points).

- The Internet hosting industry has become increasingly virtualized so that it is now common to rent a virtual machine that is run on a server farm instead of installing a physical computer. These data centers are so large (tens or hundreds of thousands of machines) that electricity is a major cost, so data centers are sometimes built in areas where electricity is cheap. This ends our quick tour of the Internet.

- We will have a great deal to say about the individual components and their design, algorithms, and protocols in subsequent chapters. One further point worth mentioning here is that what it means to be on the Internet is changing. It used to be that a machine was on the Internet if it:

  - Ran the TCP/IP protocol stack;

  - Had an IP address; and

  - Could send IP packets to all the other machines on the Internet.

- However, ISPsoften reuse IP addresses depending on which computers are in use at the moment, and home networks often share one IP address between multiple computers. This practice undermines the second condition. Security measures such as firewalls can also partly block computers from receiving packets, undermining the third condition. Despite these difficulties, it makes sense to regard such machines as being on the Internet while they are connected to their ISPs. Also worth mentioning in passing is that some companies have interconnected all their existing internal networks, often using the same technology as the Internet. These intranets are typically accessible only on company premises or from company notebooks but otherwise work the same way as the Internet.

- Draw the Architecture of 3G Mobile Networks and Explain.

- The architecture of the mobile phone network has changed greatly over the past 40 years along with its tremendous growth.

- First-generation mobile phone systems transmitted voice calls as continuously varying (analog) signals rather than sequences of (digital) bits. AMPS (Advanced Mobile Phone System), which was deployed in the United States in 1982, was a widely used first generation system.

- Second-generation mobile phone systems switched to transmitting voice calls in digital form to increase capacity, improve security, and offer text messaging. GSM (Global System for Mobile communications), which was deployed starting in 1991 and has become the most widely used mobile phone system in the world, is a 2G system.

- The third generation, or 3G, systems were initially deployed in 2001 and offer both digital voice and broadband digital data services. They also come with a lot of jargon and many different standards to choose from. 3G is loosely defined by the ITU (an international standards body we will discuss in the next section) as providing rates of at least 2 Mbps for stationary or walking users and 384 kbps in a moving vehicle.

- UMTS (Universal Mobile Telecommunications System), also called WCDMA (Wideband Code Division Multiple Access), is the main 3G system that is being rapidly deployed worldwide. It can provide up to 14 Mbps on the downlink and almost 6 Mbps on the uplink. Future releases will use multiple antennas and radios to provide even greater speeds for users. The scarce resource in 3G systems, as in 2G and 1G systems before them, is radio spectrum. Governments license the right to use parts of the spectrum to the mobile phone network operators, often using a spectrum auction in which network operators submit bids. Having a piece of licensed spectrum makes it easier to design and operate systems, since no one else is allowed transmit on that spectrum, but it often costs a serious amount of money. In the UK in 2000, for example, five 3G licenses were auctioned for a total of about $40 billion. It is the scarcity of spectrum that led to the cellular network design shown in Fig. 1-30 that is now used for mobile phone networks. To manage the radio interference between users, the coverage area is divided into cells.

- Within a cell, users are assigned channels that do not interfere with each other and do not cause too much interference for adjacent cells. This allows for good reuse of the spectrum, or frequency reuse, in the neighboring cells, which increases the capacity of the network. In 1G systems, which carried each voice call on a specific frequency band, the frequencies were carefully chosen so that they did not conflict with neighboring cells. In this way, a given frequency might only be reused once in several cells. Modern 3G systems allow each cell to use all frequencies, but in a way that results in a tolerable level of interference to the neighboring cells. There are variations on the cellular design, including the use of directional or sectored antennas on cell towers to further reduce interference, but the basic idea is the same.

Figure. Cellular design of mobile phone networks

The architecture of the mobile phone network is very different than that of the Internet. It has several parts, as shown in the simplified version of the UMTS architecture in Fig. 1-31. First, there is the air interface. This term is a fancy name for the radio communication protocol that is used over the air between the mobile device (e.g., the cell phone) and the cellular base station. Advances in the air interface over the past decades have greatly increased wireless data rates.

- The UMTS air interface is based on Code Division Multiple Access (CDMA). The cellular base station together with its controller forms the radio access network. This part is the wireless side of the mobile phone network. The controller node or RNC (Radio Network Controller) controls how the spectrum is used.

- The base station implements the air interface. It is called Node B, a temporary label that stuck. The rest of the mobile phone network carries the traffic for the radio access network. It is called the core network.

- The UMTS core network evolved from the core network used for the 2G GSM system that came before it. However, something surprising is happening in the UMTS core network. Since the beginning of networking, a war has been going on between the people who support packet networks (i.e., connectionless subnets) and the people who support circuit networks (i.e., connection-oriented subnets). The main proponents of packets come from the Internet community. In a connectionless design, every packet is routed independently of every other packet.

Figure. Architecture of the UMTS 3G mobile phone network

As a consequence, if some routers go down during a session, no harm will be done as long as the system can dynamically reconfigure itself so that subsequent packets can find some route to the destination, even if it is different from that which previous packets used.

The circuit camp comes from the world of telephone companies. In the telephone system, a caller must dial the called party’s number and wait for a connection before talking or sending data. This connection setup establishes a route through the telephone system that is maintained until the call is terminated. All words or packets follow the same route. If a line or switch on the path goes down, the call is aborted, making it less fault tolerant than a connectionless design.

The advantage of circuits is that they can support quality of service more easily. By setting up a connection in advance, the subnet can reserve resources such as link bandwidth, switch buffer space, and CPU. If an attempt is made to set up a call and insufficient resources are available, the call is rejected and the caller gets a kind of busy signal. In this way, once a connection has been set up, the connection will get good service. With a connectionless network, if too many packets arrive at the same router at the same moment, the router will choke and probably lose packets. The sender will eventually notice this and resend them, but the quality of service will be jerky and unsuitable for audio or video unless the network is lightly loaded. Needless to say, providing adequate audio quality is something telephone companies care about very much, hence their preference for connections. The surprise in Fig. is that there is both packet and circuit switched equipment in the core network. This shows the mobile phone network in transition, with mobile phone companies able to implement one or sometimes both of the alternatives. Older mobile phone networks used a circuit-switched core in the style of the traditional phone network to carry voice calls.

This legacy is seen in the UMTS network with the MSC (Mobile Switching Center), GMSC (Gate-way Mobile Switching Center), and MGW (Media Gateway) elements that set up connections over a circuit-switched core network such as the PSTN (Public Switched Telephone Network).

Data services have become a much more important part of the mobile phone network than they used to be, starting with text messaging and early packet data services such as GPRS (General Packet Radio Service) in the GSM system.

These older data services ran at tens of kbps, but users wanted more. Newer mobile phone networks carry packet data at rates of multiple Mbps. For comparison, a voice call is carried at a rate of 64 kbps, typically 3–4x less with compression. To carry all this data, the UMTS core network nodes connect directly to a packet-switched network.

The SGSN (Serving GPRS Support Node) and the GGSN (Gateway GPRS Support Node) deliver data packets to and from mobiles and interface to external packet networks such as the Internet. This transition is set to continue in the mobile phone networks that are now being planned and deployed. Internet protocols are even used on mobiles to set up connections for voice calls over a packet data network, in the manner of voiceover-IP.

IP and packets are used all the way from the radio access through to the core network. Of course, the way that IP networks are designed is also changing to support better quality of service. If it did not, then problems with chopped-up audio and jerky video would not impress paying customers. We will return to this subject in Chap. 5. Another difference between mobile phone networks and the traditional Internet is mobility. When a user moves out of the range of one cellular base station and into the range of another one, the flow of data must be re-routed from the old to the new cell base station. This technique is known as handover or handoff, and it is illustrated in Fig.

Figure. Mobile phone handover (a) before, (b) after

Either the mobile device or the base station may request a handover when the quality of the signal drops. In some cell networks, usually those based on CDMA technology, it is possible to connect to the new base station before disconnecting from the old base station. This improves the connection quality for the mobile because there is no break in service; the mobile is actually connected to two base stations for a short while. This way of doing a handover is called a soft handover to distinguish it from a hard handover, in which the mobile disconnects from the old base station before connecting to the new one.

A related issue is how to find a mobile in the first place when there is an incoming call. Each mobile phone network has a HSS (Home Subscriber Server) in the core network that knows the location of each subscriber, as well as other profile information that is used for authentication and authorization.

Starting with the 2G GSM system, the mobile phone was divided into a handset and a removable chip containing the subscriber’s identity and account information. The chip is informally called a SIM card, short for Subscriber Identity Module. SIM cards can be switched to different handsets to activate them, and they provide a basis for security. When GSM customers travel to other countries on vacation or business, they often bring their handsets but buy a new SIM card for few dollars upon arrival in order to make local calls with no roaming charges.

To reduce fraud, information on SIM cards is also used by the mobile phone network to authenticate subscribers and check that they are allowed to use the network. With UMTS, the mobile also uses the information on the SIM card to check that it is talking to a legitimate network.

Another aspect of security is privacy. Wireless signals are broadcast to all nearby receivers, so to make it difficult to eavesdrop on conversations; cryptographic keys on the SIM card are used to encrypt transmissions. This approach provides much better privacy than in 1G systems, which were easily tapped, but is not a panacea due to weaknesses in the encryption schemes.

Mobile phone networks are destined to play a central role in future networks. They are now more about mobile broadband applications than voice calls, and this has major implications for the air interfaces, core network architecture, and security of future networks. 4G technologies that are faster and better are on the drawing board under the name of LTE (Long Term Evolution), even as 3G design and deployment continues. Other wireless technologies also offer broadband Internet access to fixed and mobile clients, notably 802.16 networks under the common name of WiMAX. It is entirely possible that LTE and WiMAX are on a collision course with each other and it is hard to predict what will happen to them.

- Briefly explain about Wireless LAN : 802.11

Network connectivity to the legacy wired LAN

Desktop

with PCI 802.11 LAN card

Laptop

with PCMCIA 802.11 LAN card

- Provides network connectivity over wireless media

- An Access Point (AP) is installed to act as Bridge between Wireless and Wired Network

- The AP is connected to wired network and is equipped with antennae to provide wireless connectivity

- Range ( Distance between Access Point and WLAN client) depends on structural hindrances and RF gain of the antenna at the Access Point

- To service larger areas, multiple APs may be installed with a 20-30% overlap

- A client is always associated with one AP and when the client moves closer to another AP, it associates with the new AP (Hand-Off)

- Three flavors:

  - 802.11b

  - 802.11a

  - 802.11g

Multiple Access with Collision Avoidance (MACA)

sender

receiver

other node in

sender’s range

RTS

CTS

ACK

data

other node in

receiver’s range

- Before every data transmission

  - Sender sends a Request to Send (RTS) frame containing the length of the transmission

  - Receiver respond with a Clear to Send (CTS) frame

  - Sender sends data

  - Receiver sends an ACK; now another sender can send data

- When sender doesn’t get a CTS back, it assumes collision

WLAN: 802.11b

- The most popular 802.11 standard currently in deployment.

- Supports 1, 2, 5.5 and 11 Mbps data rates in the 2.4 GHz ISM (Industrial-Scientific-Medical) band

WLAN: 802.11a

- Operates in  the 5 GHz UNII (Unlicensed National Information Infrastructure) band

- Incompatible with devices operating in 2.4GHz

- Supports Data rates up to 54 Mbps.

WLAN: 802.11g

- Supports data rates as high as 54 Mbps on the 2.4 GHz band

- Provides backward compatibility with 802.11b equipment

- How we can model the behavior of the signal and analyze it mathematically. (Or) Discuss about the theoretical basis for data communication.

Information can be transmitted on wires by varying some physical property such as voltage or current. By representing the value of this voltage or current as a single-valued function of time, f(t), we can model the behavior of the signal and analyze it mathematically. This analysis is the subject of the following sections.

Fourier Analysis

In the early 19th century, the French mathematician Jean-Baptiste Fourier proved that any reasonably behaved periodic function, g(t) with period T, can be constructed as the sum of a (possibly infinite) number of sines and cosines:

where f 1/T is the fundamental frequency, an and bn are the sine and cosine amplitudes of the nth harmonics (terms), and c is a constant. Such a decomposition is called a Fourier series. From the Fourier series, the function can be reconstructed. That is, if the period, T, is known and the amplitudes are given, the original function of time can be found by performing the sums of Eq. (2-1).

A data signal that has a finite duration, which all of them do, can be handled by just imagining that it repeats the entire pattern over and over forever (i.e., the interval from T to 2T is the same as from 0 to T, etc.).

The an amplitudes can be computed for any given g(t) by multiplying both sides of Eq. (2-1) by sin(2πkft ) and then integrating from 0 to T. Since

only one term of the summation survives: an. The bn summation vanishes completely. Similarly, by multiplying Eq. (2-1) by cos(2kft ) and integrating between 0 and T, we can derive bn. By just integrating both sides of the equation as it stands, we can find c. The results of performing these operations are as follows:

Bandwidth-Limited Signals

The relevance of all of this to data communication is that real channels affect different frequency signals differently. Let us consider a specific example: the transmission of the ASCII character ‘‘b’’ encoded in an 8-bit byte. The bit pattern that is to be transmitted is 01100010. The left-hand part of Fig. 2-1(a) shows the voltage output by the transmitting computer. The Fourier analysis of this signal yields the coefficients:

The root-mean-square amplitudes, √an 2 bn 2 , for the first few terms are shown on the right-hand side of Fig. 2-1(a). These values are of interest because their squares are proportional to the energy transmitted at the corresponding frequency.

No transmission facility can transmit signals without losing some power in the process. If all the Fourier components were equally diminished, the resulting signal would be reduced in amplitude but not distorted [i.e., it would have the same nice squared-off shape as Fig. 2-1(a)]. Unfortunately, all transmission facilities diminish different Fourier components by different amounts, thus introducing distortion. Usually, for a wire, the amplitudes are transmitted mostly undiminished from 0 up to some frequency fc [measured in cycles/sec or Hertz (Hz)], with all frequencies above this cutoff frequency attenuated. The width of the frequency range transmitted without being strongly attenuated is called the bandwidth. In practice, the cutoff is not really sharp, so often the quoted bandwidth is from 0 to the frequency at which the received power has fallen by half.

The bandwidth is a physical property of the transmission medium that depends on, for example, the construction, thickness, and length of a wire or fiber. Filters are often used to further limit the bandwidth of a signal. 802.11 wireless channels are allowed to use up to roughly 20 MHz, for example, so 802.11 radios filter the signal bandwidth to this size. As another example, traditional (analog) television channels occupy 6 MHz each, on a wire or over the air. This filtering lets more signals share a given region of spectrum, which improves the overall efficiency of the system. It means that the frequency range for some signals will not start at zero, but this does not matter. The bandwidth is still the width of the band of frequencies that are passed, and the information that can be carried depends only on this width and not on the starting and ending frequencies. Signals that run from 0 up to a maximum frequency are called baseband signals. Signals that are shifted to occupy a higher range of frequencies, as is the case for all wireless transmissions, are called passband signals.

The signal of Fig. 2-1(a) would look if the bandwidth were so low that only the lowest frequencies were transmitted [i.e., if the function were being approximated by the first few terms of Eq. (2-1)]. Figure 2-1(b) shows the signal that results from a channel that allows only the first harmonic

(the fundamental, f) to pass through. Similarly, Fig. 2-1(c)–(e) show the spectra and reconstructed functions for higher-bandwidth channels. For digital transmission, the goal is to receive a signal with just enough fidelity to reconstruct the sequence of bits that was sent. We can already do this easily in Fig. 2-1(e), so it is wasteful to use more harmonics to receive a more accurate replica.

The Maximum Data Rate of a Channel

Nyquist proved that if an arbitrary signal has been run through a low-pass filter of bandwidth B, the filtered signal can be completely reconstructed by making only 2B (exact) samples per second. Sampling the line faster than 2B times per second is pointless because the higher-frequency components that such sampling could recover have already been filtered out. If the signal consists of V discrete levels, Nyquist’s theorem states:

maximum data rate 2B log2 V bits /sec (2-2)

For example, a noiseless 3-kHz channel cannot transmit binary (i.e., two-level) signals at a rate exceeding 6000 bps.

Shannon’s major result is that the maximum data rate or capacity of a noisy channel whose bandwidth is B Hz and whose signal-to-noise ratio is S/N, is given by:

maximum number of bits/sec B log2 (1 S/N)

Shannon’s result was derived from information-theory arguments and applies to any channel subject to thermal noise.

- Describe the different types of media that are used for transmission. (or) Explain about various transmission media. (or). Explain about Guided Transmission Media. (April 2011)(April/May 2012)(Apr 2015)

There are different types of medium used with time to improve transmission quality and rate .They are as follows

1. Guided-This has got a physical existence and its limitations in boundary.

They are also used to connect a device to other device. The conductors are metallic or glass through which the signals are transmitted in the form of electric current and light respectively. The different type are i) Twisted pair , ii) coaxial cable, iii) Fiber optic cable.

2. Unguided-No physical wired medium but through air with no boundaries there are different categories in both types of medium they are as follows

Guided cables (Wired)

Twisted –pair cable

It has two cables one to carry signal and the other for ground reference the difference of the two levels is taken by the receiving node. Cross talk and noise will be added to the two cables by the noise source. Noise is equal on both cables if they are twisted as the distance is same to both from the source of noise and unequal if parallel so cables are twisted to avoid interference from outside.

The least-expensive and most commonly-used transmission medium is twisted-pair copper wire. For over one-hundred years it has been used by telephone networks. In fact, more than 99% of the wired connections from the telephone handset to the local telephone switch use twisted-pair copper wire. Most of us have seen twisted pair in our homes and work environments. Twisted pair consists of two insulated copper wires, each about 1 mm thick, arranged in a regular spiral pattern; see Figure.  The wires are twisted together to reduce the electrical interference from similar pairs close by.  Typically, a number of pairs are bundled together in a cable by wrapping the pairs in a protective shield. A wire pair constitutes a single communication link.

Unshielded twisted pair (UTP) is commonly used for computer networks within a building, that is, for local area networks (LANs). Data rates for LANs using twisted pair today range from 10 Mbps to 100 Mbps

Coaxial cable

Here the medium is solid copper wire covered by insulator and an outer conductor which is a mesh and then finally by a plastic cover to reduce loss by interference with outside signals and called as co axial due to a common axis as shown. The different categories exist and each category is used by different applications. The

Categories and applications are as follows

  - RG-59- Cable TV.

  - RG- 58- Thin Ethernet.

  - RG-11- Thick Ethernet

Fiber-optic cable

Here the medium is a hallow glass tube through which light travels and the cladding material at which reflections take place that is at boundaries and the process of reflection is called as internal reflection for the transmission to carry on to the end of the tube that is again covered by an outer jacket as shown. Along with cross sectional view. The advantage is it provides high bandwidth and less signal attenuation. It is used as backbone of SONET.

Unguided media categories (wireless)

Here air is the medium that transmits electromagnetic signals called as wireless communication. The unguided transmissions are of different categories based on their frequencies. The different type are radio waves, micro waves and infrared with ranges as 3 KHz, 1 GHz, 1GHz-300 GHz and 300GHz-400THz respectively having their usage in multicast(radio, television), unicast(satellites, 802.11, cellular) and short range applications respectively.

- Radio frequencies allocation

- Propagation of radio waves

- Terrestrial microwave

- Satellite communication

- Cellular communication

Radio frequencies allocation

Radio transmission uses many frequencies starting at 300Hz to Table gives the frequency range and their applications. Radiofrequencies and their applications

Type Frequency Application

Propagation of radio waves

Line of sight propagation transmits exactly in the line of sight. The receive station must be in the view of the transmit station. It is sometimes called space waves or tropospheric propagation. It is limited by the curvature of the Earth for ground-based stations (100 km, from horizon to horizon). Reflected waves can cause problems. Examples of line of sight propagation are: FM radio, microwave and satellite.

Radio frequencies are in the range of 300 kHz to 10 GHz. We are seeing an emerging technology called wireless LANs. Some use radio frequencies to connect the workstations together, some use infrared technology.

Terrestrial microwave

The communication takes place between base stations on the surface of earth by travelling in air. Microwave transmission is line of sight transmission. The transmit station must be in visible contact with the receive station. This sets a limit on the distance between stations depending on the local geography. Typically the line of sight due to the Earth's curvature is only 50 km to the horizon! Repeater stations must be placed so the data signal can hop, skip and jump across the country.

Microwaves operate at high operating frequencies of 3 to 10 GHz. This allows them to carry large quantities of data due to their large bandwidth.

Satellite communication

Here the transmission is implemented using satellites. They are of different type based on the distance from earth as LEO (lower earth orbit), MEO (medium earth orbit) and GEO (geo stationery earth orbit) which rotate round the earth. Communication takes place with the earth stations receiving and transmitting information in the form of signals as uplink and down links.

Cellular communication

The transmission takes place using a base station that uses Space division multiplexing and its coverage area is called as a cell. When ever a mobile station goes out of the coverage of a station (that is cell), it has to associate itself with another station under whose coverage it lies with a handover from old station to new station(new cell).

- Discuss about various Wireless Transmission in detail.

Electromagnetic Spectrum

When electrons move, they create electromagnetic waves that can propagate through space (even in a vacuum). These waves were predicted by the British physicist James Clerk Maxwell in 1865 and first observed by the German physicist Heinrich Hertz in 1887. The number of oscillations per second of a wave is called its frequency, f, and is measured in Hz (in honor of Heinrich Hertz). The distance between two consecutive maxima (or minima) is called the wavelength, which is universally designated by the Greek letter λ (lambda).

When an antenna of the appropriate size is attached to an electrical circuit, the electromagnetic waves can be broadcast efficiently and received by a receiver some distance away. All wireless communication is based on this principle.

In a vacuum, all electromagnetic waves travel at the same speed, no matter what their frequency. This speed, usually called the speed of light, c, is approximately 3 ×108 m/sec, or about 1 foot (30 cm) per nanosecond. (A case could be made for redefining the foot as the distance light travels in a vacuum in 1 nsec rather than basing it on the shoe size of some long-dead king.) In copper or fiber the speed slows to about 2/3 of this value and becomes slightly frequency dependent. The speed of light is the ultimate speed limit. No object or signal can ever

move faster than it.

The fundamental relation between f, λ, and c (in a vacuum) is

λ f=c (2-4)

Since c is a constant, if we know f, we can find λ, and vice versa. As a rule of thumb, when λ is in meters and f is in MHz, λ f = 300. For example, 100-MHz waves are about 3 meters long, 1000-MHz waves are 0.3 meters long, and 0.1-meter waves have a frequency of 3000 MHz.

The electromagnetic spectrum is shown in Fig. 2-10. The radio, microwave, infrared, and visible light portions of the spectrum can all be used for transmitting information by modulating the amplitude, frequency, or phase of the waves. Ultraviolet light, X-rays, and gamma rays would be even better, due to their higher frequencies, but they are hard to produce and modulate, do not propagate well through buildings, and are dangerous to living things. The bands listed at the bottom of Fig. 2-10 are the official ITU (International Telecommunication Union) names and are based on the wavelengths, so the LF band goes from 1 km to 10 km (approximately 30 kHz to 300 kHz). The terms LF, MF, and HF refer to Low, Medium, and High Frequency, respectively. Clearly, when the names were assigned nobody expected to go above 10 MHz, so the higher bands were later named the Very, Ultra, Super, Extremely, and Tremendously High Frequency bands. Beyond that there are no names, but Incredibly, Astonishingly, and Prodigiously High Frequency (IHF, AHF, and PHF) would sound nice.

In frequency hopping spread spectrum, the transmitter hops from frequency to frequency hundreds of times per second. This technique is used commercially, for example, in Bluetooth and older versions of 802.11.

A second form of spread spectrum, direct sequence spread spectrum, uses a code sequence to spread the data signal over a wider frequency band. It is widely used commercially as a spectrally efficient way to let multiple signals share the same frequency band. These signals can be given different codes, a method called CDMA (Code Division Multiple Access)

A third method of communication with a wider band is UWB (Ultra-WideBand) communication. UWB sends a series of rapid pulses, varying their positions to communicate information. The rapid transitions lead to a signal that is spread thinly over a very wide frequency band. UWB is defined as signals that have a bandwidth of at least 500 MHz or at least 20% of the center frequency of their frequency band.

Radio transmission

Radio frequency (RF) waves are easy to generate, can travel long distances, and can penetrate buildings easily, so they are widely used for communication, both indoors and outdoors. Radio waves also are omnidirectional, meaning that they travel in all directions from the source, so the transmitter and receiver do not have to be carefully aligned physically.

The properties of radio waves are frequency dependent. At low frequencies, radio waves pass through obstacles well, but the power falls off sharply with distance from the source—at least as fast as 1/r 2 in air—as the signal energy is spread more thinly over a larger surface. This attenuation is called path loss. At high frequencies, radio waves tend to travel in straight lines and bounce off obstacles. Path loss still reduces power, though the received signal can depend strongly on reflections as well. High-frequency radio waves are also absorbed by rain and other obstacles to a larger extent than are low-frequency ones. At all frequencies, radio waves are subject to interference from motors and other electrical equipment.

In the VLF, LF, and MF bands, radio waves follow the ground, as illustrated in Fig. 2-12(a). These waves can be detected for perhaps 1000 km at the lower frequencies, less at the higher ones. AM radio broadcasting uses the MF band, which is why the ground waves from Boston AM radio stations cannot be heard easily in New York. Radio waves in these bands pass through buildings easily, which is why portable radios work indoors. The main problem with using these bands for data communication is their low bandwidth

Microwave Transmission

Above 100 MHz, the waves travel in nearly straight lines and can therefore be narrowly focused. Concentrating all the energy into a small beam by means of a parabolic antenna (like the familiar satellite TV dish) gives a much higher signalto- noise ratio, but the transmitting and receiving antennas must be accurately aligned with each other. In addition, this directionality allows multiple transmitters lined up in a row to communicate with multiple receivers in a row without interference, provided some minimum spacing rules are observed.

Microwave communication is so widely used for long-distance telephone communication, mobile phones, television distribution, and other purposes that a severe shortage of spectrum has developed. It has several key advantages over fiber. The main one is that no right of way is needed to lay down cables. By buying a small plot of ground every 50 km and putting a microwave tower on it, one can bypass the telephone system entirely.

Infrared Transmission

Unguided infrared waves are widely used for short-range communication. The remote controls used for televisions, VCRs, and stereos all use infrared communication. They are relatively directional, cheap, and easy to build but have a major drawback: they do not pass through solid objects. (Try standing between your remote control and your television and see if it still works.) In general, as we go from long-wave radio toward visible light, the waves behave more and more like light and less and less like radio.

On the other hand, the fact that infrared waves do not pass through solid walls well is also a plus. It means that an infrared system in one room of a building will not interfere with a similar system in adjacent rooms or buildings: you cannot control your neighbor’s television with your remote control. Furthermore, security of infrared systems against eavesdropping is better than that of radio systems precisely for this reason. Therefore, no government license is needed to operate an infrared system, in contrast to radio systems, which must be licensed outside the ISM bands. Infrared communication has a limited use on the desktop, for example, to connect notebook computers and printers with the IrDA (Infrared Data Association) standard, but it is not a major player in the communication game.

Light Transmission

Unguided optical signaling or free-space optics has been in use for centuries. Paul Revere used binary optical signaling from the Old North Church just prior to his famous ride. A more modern application is to connect the LANs in two buildings via lasers mounted on their rooftops. Optical signaling using lasers is inherently unidirectional, so each end needs its own laser and its own photodetector. This scheme offers very high bandwidth at very low cost and is relatively secure because it is difficult to tap a narrow laser beam. It is also relatively easy to install and, unlike microwave transmission, does not require an FCC license. The laser’s strength, a very narrow beam, is also its weakness here. Aiming a laser beam 1 mm wide at a target the size of a pin head 500 meters away requires the marksmanship of a latter-day Annie Oakley. Usually, lenses are put into the system to defocus the beam slightly. To add to the difficulty, wind and temperature changes can distort the beam and laser beams also cannot penetrate rain or thick fog, although they normally work well on sunny days. However, many of

these factors are not an issue when the use is to connect two spacecraft. One of the authors (AST) once attended a conference at a modern hotel in Europe at which the conference organizers thoughtfully provided a room full of terminals to allow the attendees to read their email during boring presentations.

Since the local PTT was unwilling to install a large number of telephone lines for just 3 days, the organizers put a laser on the roof and aimed it at their university’s computer science building a few kilometers away. They tested it the night before the conference and it worked perfectly. At 9 A.M. on a bright, sunny day, the link failed completely and stayed down all day. The pattern repeated itself the next two days. It was not until after the conference that the organizers discovered the problem: heat from the sun during the daytime caused convection currents to rise up from the roof of the building, as shown in Fig. 2-14. This turbulent air diverted the beam and made it dance around the detector, much like a shimmering road on

a hot day. The lesson here is that to work well in difficult conditions as well as good conditions, unguided optical links need to be engineered with a sufficient margin of error.

- Discuss about different types of Communication Satellites.

Communication satellites have some interesting properties that make them attractive for many applications. In its simplest form, a communication satellite can be thought of as a big microwave repeater in the sky. It contains several transponders, each of which listens to some portion of the spectrum, amplifies the incoming signal, and then rebroadcasts it at another frequency to avoid interference with the incoming signal. This mode of operation is known as a bent pipe. Digital processing can be added to separately manipulate or redirect data streams in the overall band, or digital information can even be received by the satellite and rebroadcast. Regenerating signals in this way improves performance compared to a bent pipe because the satellite does not amplify noise in the upward signal. The downward beams can be broad, covering a substantial fraction of the earth’s surface, or narrow, covering an area only hundreds of kilometers in diameter.

Geostationary Satellites

High-flying satellites are often called GEO (Geostationary Earth Orbit) satellites. The first geostationary satellites had a single spatial beam that illuminated about 1/3 of the earth’s surface, called its footprint. With the enormous decline in the price, size, and power requirements of microelectronics, a much more sophisticated broadcasting strategy has become possible. Each satellite is equipped with multiple antennas and multiple transponders. Each downward beam can be focused on a small geographical area, so multiple upward and downward transmissions can take place simultaneously. Typically, these so-called spot beams are elliptically shaped, and can be as small as a few hundred km in diameter.

A recent development in the communication satellite world is the development of low-cost microstations, sometimes called VSATs (Very Small Aperture Terminals)

In many VSAT systems, the microstations do not have enough power to communicate directly with one another (via the satellite, of course). Instead, a special ground station, the hub, with a large, high-gain antenna is needed to relay traffic between VSATs, as shown in Fig. 2-17. In this mode of operation, either the sender or the receiver has a large antenna and a powerful amplifier. The trade-off is a longer delay in return for having cheaper end-user stations.

Communication satellites have several properties that are radically different from terrestrial point-to-point links. To begin with, even though signals to and from a satellite travel at the speed of light (nearly 300,000 km/sec), the long round-trip distance introduces a substantial delay for GEO satellites. Depending on the distance between the user and the ground station and the elevation of the satellite above the horizon, the end-to-end transit time is between 250 and 300 msec. A typical value is 270 msec (540 msec for a VSAT system with a hub).

Satellites also have the property that the cost of transmitting a message is independent of the distance traversed. A call across the ocean costs no more to service than a call across the street. Satellites also have excellent error rates and can be deployed almost instantly, a major consideration for disaster response and military communication.

Medium-Earth Orbit Satellites

At much lower altitudes, between the two Van Allen belts, we find the MEO(Medium-Earth Orbit) satellites. As viewed from the earth, these drift slowly in longitude, taking something like 6 hours to circle the earth. Accordingly, they must be tracked as they move through the sky. Because they are lower than the GEOs, they have a smaller footprint on the ground and require less powerful transmitters to reach them. Currently they are used for navigation systems rather than telecommunications, so we will not examine them further here. The constellation of roughly 30 GPS (Global Positioning System) satellites orbiting at about 20,200 km are examples of MEO satellites.

2.4.3 Low-Earth Orbit Satellites

Moving down in altitude, we come to the LEO (Low-Earth Orbit) satellites. Due to their rapid motion, large numbers of them are needed for a complete system. On the other hand, because the satellites are so close to the earth, the ground stations do not need much power, and the round-trip delay is only a few milliseconds. The launch cost is substantially cheaper too. In this section we will examine two examples of satellite constellations for voice service, Iridium and Globalstar.

For the first 30 years of the satellite era, low-orbit satellites were rarely used because they zip into and out of view so quickly. In 1990, Motorola broke new ground by filing an application with the FCC asking for permission to launch 77 low-orbit satellites for the Iridium project (element 77 is iridium). The Iridium satellites are positioned at an altitude of 750 km, in circular polar orbits. They are arranged in north-south necklaces, with one satellite every 32 degrees of latitude, as shown in Fig. 2-18. Each satellite has a maximum of 48 cells (spot beams) and a capacity of 3840 channels, some of which are used for paging and navigation, while others are used for data and voice.

With six satellite necklaces the entire earth is covered, as suggested by Fig. 2-18. An interesting property of Iridium is that communication between distant customers takes place in space, as shown in Fig. 2-19(a). Here we see a caller at the North Pole contacting a satellite directly overhead. Each satellite has four neighbors with which it can communicate, two in the same necklace (shown) and two in adjacent necklaces (not shown). The satellites relay the call across this grid until it is finally sent down to the called at the South Pole. An alternative design to Iridium is Globalstar. It is based on 48 LEO satellites but uses a different switching scheme than that of Iridium. Whereas Iridium relays calls from satellite to satellite, which requires sophisticated switching equipment in the satellites, Globalstar uses a traditional bent-pipe design. The call originating at the North Pole in Fig. 2-19(b) is sent back to earth and picked

up by the large ground station at Santa’s Workshop. The call is then routed via a terrestrial network to the ground station nearest the callee and delivered by a bent-pipe connection as shown. The advantage of this scheme is that it puts much of the complexity on the ground, where it is easier to manage. Also, the use of large ground station antennas that can put out a powerful signal and receive a weak one means that lower-powered telephones can be used. After all, the telephone puts out only a few milliwatts of power, so the signal that gets back to the ground station is fairly weak, even after having been amplified by the satellite.

Satellites continue to be launched at a rate of around 20 per year, including ever-larger satellites that now weigh over 5000 kilograms. But there are also very small satellites for the more budget-conscious organization. To make space research more accessible, academics from Cal Poly and Stanford got together in 1999 to define a standard for miniature satellites and an associated launcher that would greatly lower launch costs (Nugent et al., 2008). CubeSats are satellites in units of 10 cm × 10 cm × 10 cm cubes, each weighing no more than 1 kilogram, that can be launched for as little as $40,000 each. The launcher flies as a secondary payload on commercial space missions. It is basically a tube that takes up to three units of cubesats and uses springs to release them into orbit. Roughly 20 cubesats have launched so far, with many more in the works. Most of them communicate with ground stations on the UHF and VHF bands.

PONDICHERRY UNIVERSITY QUESTIONS

- MARKS

- What are the two categories of physical media? Give an example for each category?(April/May 2012)  ( Ref.Qn.No.48, Pg.no.14)

- Define Network? (November 2011) ( Ref.Qn.No.2, Pg.no.4)

- What are the various types of services that the internet provides to its applications? (Apr 2013) (Ref.Qn.No.53, Pg.no.15)

- Give any two advantages of packet switching? (April/May2012)(Ref.Qn.No.47, Pg.no.14)

- What are the three categories of Access Networks?(November 2011) (Ref.Qn.No.54, Pg.no.16)

- What is twisted pair? (April 2013) (Ref.Qn.No.55, Pg.no.16)

- What is transmission delay?    (November 2011) (Ref.Qn.No.52, Pg.no.15)

- Give any two design issues that occur in computer networking.(Apr 2011) (Ref.Qn.No.45, Pg.no.14)

11 MARKS

- Short notes on connection oriented services and connectionless service? (November 2011/12) (Ref.Qn.No.5, Pg.no.29)

- Briefly explain circuit switching and packet switching? (November 2011) (April 2013) (Ref.Qn.No.6, Pg.no.31)

- Describe the different kinds of transmission media? (April 2011) (April/May 2012) (Ref.Qn.No.15, Pg.no.55)

- Draw the ISO-OSI reference model and explain the functionalities of each layer in Detail. (April/May 2012) (Ref.Qn.No.8, Pg.no.39)

- Sketch the OSI model and explain the functions of the layers that handle the following situations (Apr 2015)

- Providing control structures for communications between applications (Apr 2015) (Ref.Qn.No.7, Pg.no.37) (Session layer)

- Determining which route through the subnet to use (Apr 2015) (Ref.Qn.No.7, Pg.no.37) (network layer)

- Breaking the transmitted bit stream into packets (Apr 2015) (Ref.Qn.No.7, Pg.no.37) (network layer)

- Explain in detail about the transmission media that are commonly employed in internet . (Apr 2015) (Ref.Qn.No.15, Pg.no.55)